SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

Name	: Pranav A S

Reg : 192573001

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools. (BL4,BL5)

Assessment Tool Weightage: 30%

Annexure A

Pseudocode Writing Task (Algorithm Design)

- In a network simulation using Cisco Packet Tracer, a student records total data received as 8000 bits in 4 seconds and observes that 200 packets were sent while 180 packets were received. Write a pseudocode algorithm to calculate the network throughput in bits per second and the packet loss percentage based on the given values. The algorithm should include validation to ensure that time and total packets sent are not zero to avoid errors. It should display both the throughput and packet loss results clearly. Add logic to classify the network as Efficient if throughput is greater than 1500 bps and packet loss is less than 10%, otherwise classify it as Inefficient. Finally, ensure the algorithm outputs a complete summary of the network performance based on the calculated metrics.

- An organization has multiple network links connecting its branches. The administrator wants to monitor bandwidth utilization and identify overloaded links. Write a pseudocode algorithm that periodically collects the bandwidth usage of each network link and stores the values in an array or list. Use a loop to continuously monitor all links. Add conditions to detect when bandwidth utilization exceeds 80% and recommend switching traffic to a backup link. Explain how this algorithm supports efficient bandwidth management and prevents network congestion.

- A company has six branch offices connected through multiple network links. Each link has different bandwidth, delay, and utilization values, which change throughout the day. The network administrator wants to continuously identify the best communication path between the headquarters and each branch based on overall link quality rather than just the shortest distance.Write a pseudocode algorithm that periodically collects the bandwidth, delay, and utilization of every network link and stores the values in suitable arrays or lists. Use a loop to repeat the monitoring process at regular intervals. Calculate a link quality score for each path using the collected metrics and select the path with the highest score. Include conditions to detect when a link becomes overloaded or fails, automatically selecting an alternate path and generating an alert for the administrator. Explain how your algorithm improves network reliability, routing efficiency, and fault tolerance while adapting to changing network conditions.

Answer 1: Throughput, Packet Loss, and Performance Classification

Inputs are total data received, elapsed time, packets sent, and packets received. Outputs are throughput in bits per second, packet loss percentage, a classification, and a complete summary. The validation rules prevent division by zero and reject impossible negative values or received packets greater than sent packets.

BEGIN
    total_data_received_bits ← 8000
    elapsed_time_seconds ← 4
    packets_sent ← 200
    packets_received ← 180

    IF elapsed_time_seconds <= 0 THEN
        DISPLAY "Error: elapsed time must be greater than zero."
        STOP
    END IF

    IF packets_sent <= 0 THEN
        DISPLAY "Error: packets sent must be greater than zero."
        STOP
    END IF

    IF total_data_received_bits < 0 OR packets_received < 0 THEN
        DISPLAY "Error: data and packet values cannot be negative."
        STOP
    END IF

    IF packets_received > packets_sent THEN
        DISPLAY "Error: received packets cannot exceed sent packets."
        STOP
    END IF

    throughput_bps ← total_data_received_bits / elapsed_time_seconds
    lost_packets ← packets_sent - packets_received
    packet_loss_percent ← (lost_packets / packets_sent) * 100

    IF throughput_bps > 1500 AND packet_loss_percent < 10 THEN
        classification ← "Efficient"
    ELSE
        classification ← "Inefficient"
    END IF

    DISPLAY "NETWORK PERFORMANCE SUMMARY"
    DISPLAY "Total data received: ", total_data_received_bits, " bits"
    DISPLAY "Elapsed time: ", elapsed_time_seconds, " seconds"
    DISPLAY "Packets sent: ", packets_sent
    DISPLAY "Packets received: ", packets_received
    DISPLAY "Packets lost: ", lost_packets
    DISPLAY "Throughput: ", throughput_bps, " bps"
    DISPLAY "Packet loss: ", packet_loss_percent, "%"
    DISPLAY "Classification: ", classification
END

Interpretation and completeness: For the supplied values, throughput = 8000 / 4 = 2000 bps. Lost packets = 200 − 180 = 20, so packet loss = (20 / 200) × 100 = 10%. The throughput exceeds 1500 bps, but packet loss is not less than 10%; it is exactly 10%. Therefore, the classification is Inefficient according to the stated strict condition. The algorithm uses constant time and constant space, O(1), and handles zero, negative, and inconsistent inputs.

Rubric alignment: This solution identifies all inputs and outputs, uses clear variables and control structures, validates edge cases, provides complete pseudocode, and explains efficiency and operational behaviour.

Answer 2: Periodic Bandwidth-Utilization Monitoring

The algorithm maintains a list of link records containing an identifier, capacity, current usage, utilization percentage, state, and optional backup link. At every monitoring interval it collects fresh counters, calculates utilization, stores the values, and recommends backup routing when utilization exceeds 80%. A recovery threshold below 80% prevents repeated switching when utilization fluctuates around the threshold.

CONSTANT OVERLOAD_THRESHOLD ← 80
CONSTANT RECOVERY_THRESHOLD ← 70
CONSTANT MONITOR_INTERVAL_SECONDS ← 60

RECORD LinkStatus
    link_id
    capacity_mbps
    usage_mbps
    utilization_percent
    state
    backup_link_id
END RECORD

FUNCTION collect_bandwidth_usage(link_id)
    // Obtain the latest counters from the network-management system.
    RETURN read_usage_counter(link_id)
END FUNCTION

FUNCTION monitor_links(link_list)
    WHILE TRUE DO
        current_statuses ← EMPTY LIST

        FOR EACH link IN link_list DO
            usage ← collect_bandwidth_usage(link.link_id)

            IF link.capacity_mbps <= 0 THEN
                DISPLAY "Error: invalid capacity for ", link.link_id
                CONTINUE
            END IF

            utilization ← (usage / link.capacity_mbps) * 100
            status ← new LinkStatus
            status.link_id ← link.link_id
            status.capacity_mbps ← link.capacity_mbps
            status.usage_mbps ← usage
            status.utilization_percent ← utilization

            IF utilization > OVERLOAD_THRESHOLD THEN
                status.state ← "OVERLOADED"
                DISPLAY "ALERT: ", link.link_id, " exceeds 80% utilization."
                IF link.backup_link_id IS NOT EMPTY THEN
                    DISPLAY "Recommendation: switch eligible traffic to ", link.backup_link_id
                    recommend_traffic_shift(link.link_id, link.backup_link_id)
                ELSE
                    DISPLAY "Recommendation: apply QoS or increase link capacity."
                END IF
            ELSE IF utilization < RECOVERY_THRESHOLD THEN
                status.state ← "NORMAL"
                DISPLAY link.link_id, " has recovered below the safe threshold."
            ELSE
                status.state ← "WATCH"
                DISPLAY link.link_id, " is within the watch range."
            END IF

            APPEND status TO current_statuses
            store_status_with_timestamp(status)
        END FOR

        DISPLAY current_statuses
        SLEEP FOR MONITOR_INTERVAL_SECONDS
    END WHILE
END FUNCTION

BEGIN
    links ← load_configured_links_and_backup_relationships()
    monitor_links(links)
END

Interpretation and completeness: The loop gives the administrator a continuous view of the network rather than a single measurement. The 80% threshold identifies links at risk of congestion, while the lower recovery threshold reduces oscillation between primary and backup paths. Persisting timestamped records supports trend analysis, capacity planning, and post-incident investigation. The algorithm is O(L) per monitoring cycle for L links, uses an array or list for current measurements, and can be extended with hysteresis, alerts, QoS policies, and automatic routing changes subject to administrator approval.

Rubric alignment: This solution identifies all inputs and outputs, uses clear variables and control structures, validates edge cases, provides complete pseudocode, and explains efficiency and operational behaviour.

Answer 3: Adaptive Best-Path Selection Using Link Quality

The algorithm evaluates paths using bandwidth, delay, utilization, and availability instead of distance alone. Metrics are normalized against configured limits so that a high bandwidth contributes positively while high delay and high utilization reduce the score. The weights can be tuned to the organisation’s priorities; the example gives bandwidth 40%, delay 25%, utilization 25%, and availability 10%. Overloaded or failed links are excluded, and an alert is generated whenever the active path changes or becomes unusable.

CONSTANT BANDWIDTH_WEIGHT ← 0.40
CONSTANT DELAY_WEIGHT ← 0.25
CONSTANT UTILIZATION_WEIGHT ← 0.25
CONSTANT AVAILABILITY_WEIGHT ← 0.10
CONSTANT OVERLOAD_THRESHOLD ← 80
CONSTANT MONITOR_INTERVAL_SECONDS ← 30

RECORD LinkMetric
    link_id
    source
    destination
    bandwidth_mbps
    delay_ms
    utilization_percent
    is_up
    quality_score
END RECORD

FUNCTION calculate_quality(link, maximum_bandwidth, maximum_delay)
    IF link.is_up = FALSE OR link.utilization_percent >= OVERLOAD_THRESHOLD THEN
        RETURN -1
    END IF

    bandwidth_score ← MIN(link.bandwidth_mbps / maximum_bandwidth, 1)
    delay_score ← 1 - MIN(link.delay_ms / maximum_delay, 1)
    utilization_score ← 1 - (link.utilization_percent / 100)
    availability_score ← 1

    RETURN (BANDWIDTH_WEIGHT * bandwidth_score) +
           (DELAY_WEIGHT * delay_score) +
           (UTILIZATION_WEIGHT * utilization_score) +
           (AVAILABILITY_WEIGHT * availability_score)
END FUNCTION

FUNCTION discover_all_paths(source, destination, topology)
    // Return every loop-free candidate path between the two nodes.
    RETURN enumerate_paths(source, destination, topology)
END FUNCTION

FUNCTION select_best_path(source, destination, topology, link_limits)
    candidate_paths ← discover_all_paths(source, destination, topology)
    best_path ← EMPTY
    best_score ← -1

    FOR EACH path IN candidate_paths DO
        path_score ← 0
        path_is_valid ← TRUE

        FOR EACH link IN path DO
            metric ← read_current_metric(link.link_id)
            metric.quality_score ← calculate_quality(
                metric, link_limits.maximum_bandwidth, link_limits.maximum_delay)
            store_metric(metric)

            IF metric.quality_score = -1 THEN
                path_is_valid ← FALSE
                BREAK
            END IF

            // The weakest link limits the practical quality of the path.
            path_score ← MIN(path_score, metric.quality_score)
                IF path_score = 0 THEN
                    path_score ← metric.quality_score
                END IF
        END FOR

        IF path_is_valid AND path_score > best_score THEN
            best_score ← path_score
            best_path ← path
        END IF
    END FOR

    RETURN best_path, best_score
END FUNCTION

FUNCTION monitor_and_route(branches, topology, link_limits)
    active_paths ← EMPTY MAP

    WHILE TRUE DO
        FOR EACH branch IN branches DO
            previous_path ← active_paths[branch]
            path, score ← select_best_path("HEADQUARTERS", branch, topology, link_limits)

            IF path IS EMPTY THEN
                DISPLAY "CRITICAL ALERT: no usable path to ", branch
                generate_alert(branch, "No available path")
                CONTINUE
            END IF

            IF previous_path IS EMPTY OR path != previous_path THEN
                active_paths[branch] ← path
                install_route(branch, path)
                generate_alert(branch, "Path changed to an alternate or better path")
            END IF

            DISPLAY "Branch: ", branch, " Selected path: ", path, " Score: ", score
        END FOR

        SLEEP FOR MONITOR_INTERVAL_SECONDS
    END WHILE
END FUNCTION

BEGIN
    branches ← ["BRANCH_1", "BRANCH_2", "BRANCH_3", "BRANCH_4", "BRANCH_5", "BRANCH_6"]
    topology ← load_network_topology()
    link_limits ← load_normalization_limits()
    monitor_and_route(branches, topology, link_limits)
END

Interpretation and completeness: The score rewards high available bandwidth and availability while penalizing delay and utilization. A failed or overloaded link receives a score of −1 and cannot be selected. The weakest-link rule prevents a path with one severely degraded segment from appearing better merely because its other links are strong. Continuous monitoring adapts routes as traffic changes, and alerts make failover visible to the administrator. If P candidate paths are evaluated and each contains at most K links, one cycle requires O(PK) metric processing, with storage O(L) for L monitored links plus the selected-path table. In production, route changes should include hysteresis or a minimum hold time to prevent excessive oscillation.

Rubric alignment: This solution identifies all inputs and outputs, uses clear variables and control structures, validates edge cases, provides complete pseudocode, and explains efficiency and operational behaviour.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |