SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

# COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

# Annexure A SHORT ANSWER TEST

Code of Conduct:

I ____Pranav A S 192573001________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Pranav A S

Annexure A

# Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum

Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers:

Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoTenabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

# RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, wellexplained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Wellstructured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| --- | --- | --- | --- | --- | --- |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

ANSWERS

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Given:

• File size = 150 MB = 150,000,000 bytes

• Transport Layer segment size = 1500 bytes

• Data Link Layer overhead = 40 bytes per frame

• Bandwidth = 50 Mbps = 50,000,000 bits/second

• Propagation delay is ignored

Step 1: Calculate the Number of Transport Layer Segments

Number of Segments = File Size ÷ Segment Size

= 150,000,000 ÷ 1500

= 100,000 segments

Answer: 100,000 segments

Step 2: Calculate the Number of Data Link Layer Frames

Each segment is encapsulated into one frame.

Number of Frames = 100,000

Answer: 100,000 frames

Step 3: Calculate the Total Data Link Layer Overhead

Total Overhead = Number of Frames × Overhead per Frame

= 100,000 × 40

= 4,000,000 bytes

Answer: 4,000,000 bytes (4 MB)

Step 4: Calculate the Total Data Transmitted

Total Data = File Size + Overhead

= 150,000,000 + 4,000,000

= 154,000,000 bytes

Convert to bits

154,000,000 × 8 = 1,232,000,000 bits

Step 5: Calculate the Transmission Time

Transmission Time = Total Bits ÷ Bandwidth

= 1,232,000,000 ÷ 50,000,000

= 24.64 seconds

Answer: 24.64 seconds

Final Answers:

• Number of Transport Layer Segments = 100,000

• Number of Data Link Layer Frames = 100,000

• Total Data Link Layer Overhead = 4,000,000 bytes (4 MB)

• Total Data Transmitted = 154,000,000 bytes

• Estimated Transmission Time = 24.64 seconds

How the Transport and Data Link Layers Ensure Reliable Communication

Transport Layer:

• Divides the file into segments.

• Assigns sequence numbers to each segment.

• Uses acknowledgments (ACKs) for successful delivery.

• Retransmits lost or damaged segments.

• Performs flow control to avoid receiver overload.

• Detects errors using checksums.

Data Link Layer:

• Encapsulates each segment into a frame.

• Adds a 40-byte header and trailer.

• Detects transmission errors using CRC (Cyclic Redundancy Check).

• Discards corrupted frames and may request retransmission.

• Controls access to the communication medium using MAC protocols.

Conclusion:

The Transport Layer provides reliable end-to-end communication between the client and the remote server, while the Data Link Layer ensures reliable transmission of each frame over individual network links.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Given:

• Sliding window size = 6 frames

• Frame size = 1024 bytes

• Total frames transmitted = 48

Step 1: Calculate the Number of Transmission Windows

Number of Transmission Windows = Total Frames ÷ Window Size

= 48 ÷ 6

= 8 windows

Answer: 8 windows

Step 2: Calculate the Total Number of Retransmissions

One frame is lost in every transmission window.

Total Retransmissions = Number of Windows × 1

= 8 × 1

= 8 frames

Answer: 8 retransmissions

Step 3: Calculate the Total User Data Transmitted

Total User Data = Number of Frames × Frame Size

= 48 × 1024

= 49,152 bytes

Answer: 49,152 bytes

Final Answers:

• Number of Transmission Windows = 8

• Total Retransmissions = 8 frames

• Total User Data Transmitted = 49,152 bytes

How Flow Control Improves Communication Efficiency

Transport Layer:

• Controls the rate of data transmission between the sender and receiver.

• Prevents the receiver's buffer from overflowing.

• Ensures the sender transmits data only at the rate the receiver can process.

• Reduces packet loss and unnecessary retransmissions.

• Improves network throughput and communication reliability.

Conclusion:

The Transport Layer uses flow control to match the sender's transmission speed with the receiver's processing capacity. This prevents congestion, minimizes retransmissions, and ensures efficient and reliable communication.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Given:

• IP Packet Size = 12,000 bytes

• Maximum Transmission Unit (MTU) = 1500 bytes

• IP Header Size = 20 bytes per fragment

Step 1: Calculate the Maximum Data in Each Fragment

Maximum Data per Fragment = MTU − IP Header

= 1500 − 20

= 1480 bytes

Answer: 1480 bytes

Step 2: Calculate the Number of Fragments

Number of Fragments = 12,000 ÷ 1480

= 8.11

Since a fraction of a fragment cannot be transmitted,

Number of Fragments = 9 fragments

Answer: 9 fragments

Step 3: Calculate the Total Header Overhead

Total Header Overhead = Number of Fragments × Header Size

= 9 × 20

= 180 bytes

Answer: 180 bytes

Final Answers:

• Number of Fragments = 9

• Total Header Overhead = 180 bytes

How the Network Layer Ensures Successful Delivery

Network Layer:

• Divides large packets into smaller fragments according to the MTU.

• Assigns identification numbers to each fragment.

• Uses fragment offsets for correct ordering.

• Routes each fragment independently through the network.

• Reassembles all fragments correctly at the destination.

Conclusion:

The Network Layer ensures successful packet delivery by fragmenting large packets, routing them efficiently, and reassembling them correctly at the destination.

4. A Transport efficiency.Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication

Given:

• Sliding window size = 6 frames

• Frame size = 1024 bytes

• Total frames transmitted = 48

Step 1: Calculate the Number of Transmission Windows

Number of Transmission Windows = Total Frames ÷ Window Size

= 48 ÷ 6

= 8 windows

Answer: 8 windows

Step 2: Calculate the Total Number of Retransmissions

One frame is lost in every transmission window.

Total Retransmissions = Number of Windows × 1

= 8 × 1

= 8 frames

Answer: 8 retransmissions

Step 3: Calculate the Total User Data Transmitted

Total User Data = Number of Frames × Frame Size

= 48 × 1024

= 49,152 bytes

Answer: 49,152 bytes

Final Answers:

• Number of Transmission Windows = 8

• Total Retransmissions = 8 frames

• Total User Data Transmitted = 49,152 bytes

How Flow Control Improves Communication Efficiency

Transport Layer:

• Controls the rate of data transmission between the sender and receiver.

• Prevents the receiver's buffer from overflowing.

• Ensures the sender transmits data at the receiver's processing speed.

• Reduces packet loss and unnecessary retransmissions.

• Improves communication efficiency and network throughput.

Conclusion:

The Transport Layer uses flow control to regulate the rate of data transmission between the sender and receiver. This prevents congestion, minimizes retransmissions, improves throughput, and ensures reliable communication.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Given:

• Video file size = 600 MB

• Synchronization checkpoint after every 60 MB

• Failure occurs after 390 MB

Step 1: Calculate the Number of Checkpoints Created Before Failure

Number of Checkpoints = 390 ÷ 60

= 6.5

Only completed checkpoints are counted.

Completed Checkpoints = 6

Answer: 6 checkpoints

Step 2: Calculate the Amount of Data to be Retransmitte

Last Completed Checkpoint = 6 × 60

= 360 MB

Data to be Retransmitted = 390 − 360

= 30 MB

Answer: 30 MB

Final Answers:

• Number of Checkpoints Created = 6

• Amount of Data to be Retransmitted = 30 MB

How the Session Layer Ensures Reliable Communication

Session Layer:

• Establishes and maintains communication sessions.

• Inserts synchronization checkpoints during data transfer.

• Allows communication to resume from the last checkpoint after a failure.

• Prevents retransmission of the entire file.

• Saves transmission time and network bandwidth.

Conclusion:

The Session Layer improves communication reliability by using synchronization checkpoints, allowing interrupted data transfers to resume from the last saved checkpoint instead of restarting from the beginning.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Given:

• Original file size = 900 MB

• Compression = 40%

• Encryption overhead = 5%

Step 1: Calculate the Compressed File Size

Compressed File Size = 900 × (100 − 40)%

= 900 × 60%

= 540 MB

Answer: 540 MB

Step 2: Calculate the Encrypted File Size

Encrypted File Size = 540 × 105%

= 567 MB

Answer: 567 MB

Step 3: Calculate the Total Percentage Reduction

Percentage Reduction = ((900 − 567) ÷ 900) × 100

= (333 ÷ 900) × 100

= 37%

Answer: 37%

Final Answers:

• Compressed File Size = 540 MB

• Encrypted File Size = 567 MB

• Total Percentage Reduction = 37%

How the Presentation Layer Improves Transmission Efficiency and Security

Presentation Layer:

• Compresses data to reduce transmission size.

• Encrypts data to provide confidentiality.

• Converts data into a standard format.

• Ensures compatibility between different systems.

• Enhances security during data transmission.

Conclusion:

The Presentation Layer improves communication by reducing file size through compression and protecting data using encryption, resulting in efficient and secure data transmission.

7. An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Given:

• Data size = 3 GB = 3000 MB

• Request size = 3 MB

• Throughput = 120 Mbps

Step 1: Calculate the Number of Requests

Number of Requests = Data Size ÷ Request Size

= 3000 ÷ 3

= 1000 requests

Answer: 1000 requests

Step 2: Calculate the Total Data in Megabits

Total Data = 3000 × 8

= 24,000 Mb

Answer: 24,000 Mb

Step 3: Calculate the Transmission Time

Transmission Time = Total Data ÷ Throughput

= 24,000 ÷ 120

= 200 seconds

Answer: 200 seconds

Final Answers:

• Number of Requests = 1000

• Estimated Transmission Time = 200 seconds

How the Application Layer Provides Reliable Services

Application Layer:

• Provides file transfer services using FTP.

• Supports user authentication.

• Manages file transfer requests.

• Reports communication errors.

• Provides an interface between users and network services.

Conclusion:

The Application Layer provides reliable services by managing file transfers, handling user requests, and ensuring successful communication between applications over the network.

8. A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Given:

• Application Layer Delay = 4 ms

• Transport Layer Delay = 3 ms

• Network Layer Delay = 5 ms

• Data Link Layer Delay = 2 ms

• Physical Layer Delay = 1 ms

• Propagation Delay = 18 ms

• Transmission Delay = 12 ms

Step 1: Calculate the Total Processing Delay

Total Processing Delay = 4 + 3 + 5 + 2 + 1

= 15 ms

Answer: 15 ms

Step 2: Calculate the Total End-to-End Delay

Total End-to-End Delay = Processing Delay + Propagation Delay + Transmission Delay

= 15 + 18 + 12

= 45 ms

Answer: 45 ms

Final Answers:

• Total Processing Delay = 15 ms

• Total End-to-End Delay = 45 ms

How Each OSI Layer Contributes to Communication

Application Layer:

• Provides services to user applications.

• Enables communication between network applications.

Transport Layer:

• Ensures reliable end-to-end data delivery.

• Performs error checking and flow control.

Network Layer:

• Determines the best route for packet transmission.

• Handles logical addressing and routing.

Data Link Layer:

• Performs framing and error detection.

• Ensures reliable node-to-node communication.

Physical Layer:

• Transmits data as electrical, optical, or radio signals.

• Transfers bits through the communication medium.

Conclusion:

Each OSI layer performs a specific function in the communication process. Together, they ensure reliable, efficient, and accurate data transmission with a total end-to-end delay of 45 ms.

9. A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Given:

• Useful Data = 80 MB = 80,000,000 bytes

• Frame Size = 1600 bytes

• Protocol Overhead = 80 bytes per frame

Step 1: Calculate the Useful Data in Each Frame

Useful Data per Frame = Frame Size − Protocol Overhead

= 1600 − 80

= 1520 bytes

Answer: 1520 bytes

Step 2: Calculate the Number of Frames Required

Number of Frames = Useful Data ÷ Useful Data per Frame

= 80,000,000 ÷ 1520

= 52,631.58

Since a fraction of a frame cannot be transmitted,

Number of Frames = 52,632 frames

Answer: 52,632 frames

Step 3: Calculate the Total Protocol Overhead

Total Overhead = Number of Frames × Protocol Overhead

= 52,632 × 80

= 4,210,560 bytes

Answer: 4,210,560 bytes

Step 4: Calculate the Transmission Efficiency

Transmission Efficiency = (Useful Data per Frame ÷ Frame Size) × 100

= (1520 ÷ 1600) × 100

= 95%

Answer: 95%

Final Answers:

• Number of Frames Required = 52,632

• Total Protocol Overhead = 4,210,560 bytes

• Transmission Efficiency = 95%

How Protocol Overhead Affects Network Performance

Protocol Overhead:

• Adds headers and trailers required for communication.

• Ensures error detection and reliable data transfer.

• Increases the total amount of transmitted data.

• Consumes additional bandwidth.

• Slightly reduces overall transmission efficiency.

Conclusion:

Protocol overhead is essential for reliable communication because it provides addressing, error detection, and control information. However, it increases bandwidth usage and slightly reduces transmission efficiency. In this case, the network achieves an efficiency of 95%.

10. A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Given:

• Original File Size = 240 MB

• Compression = 30%

• Segment Size = 1400 bytes

• Data Link Layer Overhead = 60 bytes per frame

Step 1: Calculate the Compressed File Size

Compressed File Size = 240 × (100 − 30)%

= 240 × 70%

= 168 MB

Answer: 168 MB

Step 2: Convert the Compressed File Size into Bytes

Compressed File Size = 168 × 1,000,000

= 168,000,000 bytes

Step 3: Calculate the Number of Segments

Number of Segments = 168,000,000 ÷ 1400

= 120,000 segments

Answer: 120,000 segments

Step 4: Calculate the Total Protocol Overhead

Total Protocol Overhead = Number of Segments × Overhead per Frame

= 120,000 × 60

= 7,200,000 bytes

Answer: 7,200,000 bytes

Step 5: Calculate the Final Data Transmitted Across the Physical Layer

Final Data Transmitted = Compressed Data + Total Protocol Overhead

= 168,000,000 + 7,200,000

= 175,200,000 bytes

Answer: 175,200,000 bytes

Final Answers:

• Compressed File Size = 168 MB

• Number of Segments Transmitted = 120,000

• Total Protocol Overhead = 7,200,000 bytes

• Final Data Transmitted Across the Physical Layer = 175,200,000 bytes

How Multiple OSI Layers Work Together

Presentation Layer:

• Compresses the medical record to reduce transmission size.

• Encrypts and formats data for secure communication.

Transport Layer:

• Divides the compressed data into segments.

• Ensures reliable delivery using error checking and flow control.

Data Link Layer:

• Adds frame headers and trailers.

• Performs error detection for reliable node-to-node communication.

Physical Layer:

• Transmits the final data as electrical, optical, or wireless signals through the communication medium.

Conclusion:

Multiple OSI layers work together to provide secure, reliable, and efficient communication. The Presentation Layer compresses the data, the Transport Layer segments it for reliable delivery, the Data Link Layer adds protocol information for error detection, and the Physical Layer transmits the complete data across the network.