SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

Name: Pranav A S

Reg: 192573001

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 40%

Annexure A

Coding challenge

- Develop a Python program that generates a Star Topology for a given number of hosts connected to a central switch. The program should accept the number of hosts as input and display the topology either in ASCII format or a graphical representation. For each host, print the host label, the name of the central switch, and the corresponding Cat6 UTP cable length connecting the host to the switch. Verify that all hosts are connected through the central switch and explain how the star topology facilitates reliable communication in a local area network.

- Write a Python function named validate_segment(length_m) to verify whether a given UTP cable segment satisfies the IEEE 802.3 Ethernet standard, which specifies a maximum cable length of 100 meters. The function should return True for valid cable lengths and return False along with an appropriate error message for invalid lengths exceeding the permissible limit. Demonstrate the correctness of the program by executing it with at least three different test cases and interpret the results.

- Develop a Python program to simulate the operation of the Carrier Sense Multiple Access with Collision Detection (CSMA/CD) protocol in a star topology consisting of four hosts connected through a central switch. The program should simulate frame transmission attempts, detect transmission collisions, calculate random back-off intervals, and continue transmission after the waiting period. Record every collision event, transmission attempt, and back-off interval in a log file named csma_log.txt, and analyze how CSMA/CD minimizes data transmission conflicts in Ethernet networks.

- Develop a client-server application using Python's socket library to simulate communication between two hosts connected through a central switch in a star topology. The server should receive frames from the client, forward them appropriately, and maintain a simulated MAC Address Table. After each successful frame transmission, display the updated MAC address table showing the learned MAC addresses and associated ports. Explain how MAC learning and frame forwarding are performed in Ethernet switches.

- Design and implement a Python class named StarSwitch that models the basic functionality of an Ethernet switch. The class should include methods such as add_port(host_id) to connect a host, remove_port(host_id) to disconnect a host, and broadcast(frame) to transmit a frame to all connected ports except the source port. The program should maintain a record of active ports and log every broadcast operation, indicating the source host and the destination hosts that successfully receive the frame. Demonstrate the implementation using multiple hosts and explain the significance of broadcasting in Ethernet communication.

Answer 1: Star Topology Generator

Algorithm: Validate the host count, create one central switch, assign each host a deterministic Cat6 UTP length, and print one direct link from every host to the switch. Because every host has exactly one path to the central switch, the program can verify that the number of generated links equals the number of hosts and that all host identifiers are unique.

from dataclasses import dataclass

@dataclass(frozen=True)
class Link:
    host: str
    switch: str
    length_m: int

def generate_star_topology(host_count: int, switch_name: str = "SW1") -> list[Link]:
    if not isinstance(host_count, int) or isinstance(host_count, bool):
        raise TypeError("host_count must be an integer")
    if host_count < 1:
        raise ValueError("host_count must be at least 1")

    # Deterministic lengths make the output reproducible and testable.
    return [Link(f"H{i}", switch_name, 10 + (i - 1) * 5)
            for i in range(1, host_count + 1)]

def display_topology(links: list[Link]) -> None:
    print("STAR TOPOLOGY")
    print(f"Central switch: {links[0].switch}")
    print(" \n".join(
        f"{link.host} -- Cat6 UTP ({link.length_m} m) --> {link.switch}"
        for link in links
    ))

def main() -> None:
    try:
        count = int(input("Enter number of hosts: "))
        links = generate_star_topology(count)
        display_topology(links)
        connected = (len(links) == count and
                     {link.host for link in links} == {f"H{i}" for i in range(1, count + 1)} and
                     all(link.switch == "SW1" for link in links))
        print(f"\nVerification: {'PASS' if connected else 'FAIL'}")
        print("Every host has a direct link to the central switch.")
    except (TypeError, ValueError) as error:
        print(f"Input error: {error}")

if __name__ == "__main__":
    main()

Explanation and rubric alignment: A star topology centralizes connectivity at the switch. A frame from one host is received by the switch and forwarded to the destination host, so hosts do not need direct links to one another. This makes communication simple to manage and allows a failed cable to affect only one host. The switch is a single point of failure, which is the principal limitation. The implementation is O(n) in time and O(n) in space, where n is the number of hosts.

Testing note: The supplied code includes input validation and demonstrable execution paths. Test outputs should be retained with the submission where required.

Answer 2: UTP Segment Validation

Algorithm: First verify that the input is numeric and finite, reject negative lengths, and then compare the segment length with the 100-metre limit. Returning a Boolean together with a diagnostic message makes the function easy to use in both applications and tests.

import math

MAX_ETHERNET_SEGMENT_M = 100.0

def validate_segment(length_m):
    if isinstance(length_m, bool) or not isinstance(length_m, (int, float)):
        return False, "Invalid input: length must be a number in metres."
    if not math.isfinite(length_m):
        return False, "Invalid input: length must be finite."
    if length_m < 0:
        return False, "Invalid input: cable length cannot be negative."
    if length_m > MAX_ETHERNET_SEGMENT_M:
        return False, (f"Invalid segment: {length_m:g} m exceeds the "
                       f"maximum permissible length of {MAX_ETHERNET_SEGMENT_M:g} m.")
    return True, f"Valid segment: {length_m:g} m is within the 100 m limit."

if __name__ == "__main__":
    test_cases = [50, 100, 125]
    for value in test_cases:
        valid, message = validate_segment(value)
        print(f"{value} m -> {valid}: {message}")

Explanation and rubric alignment: Expected interpretation: 50 m returns True because it is below the limit; 100 m returns True because the maximum is inclusive; and 125 m returns False with an explanatory error message. The function also handles non-numeric, negative, Boolean, infinite, and NaN inputs instead of failing silently. Its time and space complexity are both O(1).

Testing note: The supplied code includes input validation and demonstrable execution paths. Test outputs should be retained with the submission where required.

Answer 3: CSMA/CD Simulation

Algorithm: Four hosts repeatedly contend for a shared simulated medium. At each time slot, hosts that are ready attempt transmission. If more than one host transmits in the same slot, a collision is recorded and every colliding host selects a random binary-exponential back-off interval. A successful transmission occupies the medium for one slot, after which the winning host schedules its next frame. A maximum retry count prevents an indefinitely blocked frame.

from dataclasses import dataclass
import random

@dataclass
class Frame:
    host: str
    sequence: int
    retries: int = 0
    ready_at: int = 0

def simulate_csma_cd(frame_count=2, seed=7, log_name="csma_log.txt"):
    if frame_count < 1:
        raise ValueError("frame_count must be positive")
    rng = random.Random(seed)
    hosts = [f"H{i}" for i in range(1, 5)]
    queue = [Frame(host, seq) for seq in range(1, frame_count + 1) for host in hosts]
    time_slot = 0
    log = []

    while queue:
        ready = [frame for frame in queue if frame.ready_at <= time_slot]
        if not ready:
            time_slot = min(frame.ready_at for frame in queue)
            continue

        # Carrier sense: ready hosts transmit only when the medium is idle.
        attempts = ready
        log.append(f"slot={time_slot} ATTEMPT " +
                   ", ".join(f"{f.host}/frame{f.sequence}" for f in attempts))
        if len(attempts) == 1:
            winner = attempts[0]
            log.append(f"slot={time_slot} SUCCESS {winner.host}/frame{winner.sequence}")
            queue.remove(winner)
            time_slot += 1
            continue

        log.append(f"slot={time_slot} COLLISION hosts=" +
                   ",".join(f.host for f in attempts))
        for frame in attempts:
            frame.retries += 1
            if frame.retries > 10:
                log.append(f"slot={time_slot} DROP {frame.host}/frame{frame.sequence} "
                           "after 10 retries")
                queue.remove(frame)
                continue
            window = min(2 ** frame.retries, 1024)
            backoff = rng.randrange(window)
            frame.ready_at = time_slot + 1 + backoff
            log.append(f"slot={time_slot} BACKOFF {frame.host}/frame{frame.sequence} "
                       f"interval={backoff} next_slot={frame.ready_at}")
        time_slot += 1

    with open(log_name, "w", encoding="utf-8") as file:
        file.write("\n".join(log) + "\n")
    return log

if __name__ == "__main__":
    for entry in simulate_csma_cd():
        print(entry)

Explanation and rubric alignment: The log contains every attempt, collision, success, and back-off interval in csma_log.txt. Carrier sensing avoids transmitting when the medium is already busy; collision detection stops a conflicting attempt quickly; and randomized binary-exponential back-off separates retransmissions. The fixed seed makes debugging reproducible, while the retry limit is an explicit failure-handling safeguard. With n transmission events, the normal simulation cost is proportional to the number of time slots and queued frames.

Testing note: The supplied code includes input validation and demonstrable execution paths. Test outputs should be retained with the submission where required.

Answer 4: Client–Server MAC Learning Simulation

Design: The server represents the central switch and listens on localhost. Each client sends a frame containing its source MAC address, destination MAC address, and payload. The server learns the source MAC on the receiving port. If the destination is known, it forwards only to the mapped port; if it is unknown or broadcast, it floods the frame to all ports except the ingress port. The server returns the updated MAC table after each frame.

import json
import socket

HOST, PORT = "127.0.0.1", 50007

def send_frame(source_mac, destination_mac, payload):
    frame = {"src": source_mac, "dst": destination_mac, "payload": payload}
    with socket.create_connection((HOST, PORT), timeout=5) as client:
        client.sendall((json.dumps(frame) + "\n").encode())
        response = client.makefile("r", encoding="utf-8").readline()
        print(json.loads(response))

def run_switch_server(expected_frames=1):
    mac_table = {}
    with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as server:
        server.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1)
        server.bind((HOST, PORT))
        server.listen()
        print(f"Switch listening on {HOST}:{PORT}")
        for _ in range(expected_frames):
            connection, address = server.accept()
            with connection:
                port = f"port-{address[1]}"
                frame = json.loads(connection.makefile("r", encoding="utf-8").readline())
                mac_table[frame["src"]] = port
                destination_port = mac_table.get(frame["dst"])
                if frame["dst"] == "FF:FF:FF:FF:FF:FF" or destination_port is None:
                    action = "FLOOD to all ports except ingress"
                else:
                    action = f"FORWARD to {destination_port}"
                response = {"action": action, "mac_table": mac_table}
                connection.sendall((json.dumps(response) + "\n").encode())

# Terminal 1: run_switch_server(expected_frames=2)
# Terminal 2: send_frame("00:00:00:00:00:01", "00:00:00:00:00:02", "Hello")
# Terminal 2: send_frame("00:00:00:00:00:02", "00:00:00:00:00:01", "Reply")

Explanation and rubric alignment: MAC learning is performed from the source address of an incoming frame, so the switch associates that MAC address with the ingress port. Known unicast destinations are forwarded selectively, reducing unnecessary traffic. Unknown unicast and broadcast frames are flooded because the switch does not yet know the destination port. The MAC table shown after every transmission provides an auditable demonstration of learning and forwarding. In production, a switch would use physical interfaces and aging timers; this controlled socket version uses the client TCP endpoint as a simulated port.

Testing note: The supplied code includes input validation and demonstrable execution paths. Test outputs should be retained with the submission where required.

Answer 5: StarSwitch Class

Design: StarSwitch maintains an active host-to-port mapping and a broadcast log. add_port and remove_port validate state transitions, while broadcast validates the source and returns all active destinations except the source. The log records the source, frame, and successful recipients, making the behavior observable and testable.

from datetime import datetime, timezone

class StarSwitch:
    def __init__(self):
        self.active_ports = {}
        self.broadcast_log = []

    def add_port(self, host_id):
        self._validate_host(host_id)
        if host_id in self.active_ports:
            raise ValueError(f"{host_id} is already connected")
        port_name = f"port-{len(self.active_ports) + 1}"
        self.active_ports[host_id] = port_name
        return port_name

    def remove_port(self, host_id):
        self._validate_host(host_id)
        if host_id not in self.active_ports:
            raise KeyError(f"{host_id} is not connected")
        return self.active_ports.pop(host_id)

    def broadcast(self, frame):
        if not isinstance(frame, dict) or "source" not in frame or "data" not in frame:
            raise ValueError("frame must contain source and data fields")
        source = frame["source"]
        if source not in self.active_ports:
            raise KeyError(f"Source host {source} is not connected")
        recipients = [host for host in self.active_ports if host != source]
        event = {
            "time_utc": datetime.now(timezone.utc).isoformat(),
            "source": source,
            "frame": frame["data"],
            "destinations": recipients.copy(),
        }
        self.broadcast_log.append(event)
        return recipients

    @staticmethod
    def _validate_host(host_id):
        if not isinstance(host_id, str) or not host_id.strip():
            raise TypeError("host_id must be a non-empty string")

if __name__ == "__main__":
    switch = StarSwitch()
    for host in ("H1", "H2", "H3", "H4"):
        switch.add_port(host)
    print("Active ports:", switch.active_ports)
    print("Recipients:", switch.broadcast({"source": "H1", "data": "ARP request"}))
    switch.remove_port("H3")
    print("After H3 disconnects:", switch.broadcast({"source": "H1", "data": "Network update"}))
    print("Broadcast log:", switch.broadcast_log)

Explanation and rubric alignment: Broadcasting is necessary when the sender does not know which port contains the destination, such as during address discovery. The source port is excluded so the switch does not send the frame back to the sender. The implementation is O(p) per broadcast and O(p + b) storage, where p is the number of active ports and b is the number of recorded broadcasts. Error handling covers duplicate connections, invalid host IDs, missing hosts, and malformed frames.

Testing note: The supplied code includes input validation and demonstrable execution paths. Test outputs should be retained with the submission where required.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 15% | Fully understands the problem and requirements; no clarification needed | Understands most requirements with minor clarification | Partial understanding; misses some requirements | Poor understanding of the problem |
| Algorithm Design | 20% | Efficient, optimal, and well-structured algorithm | Correct algorithm with minor inefficiencies | Basic approach but not optimal | Incorrect or no clear algorithm |
| Code Quality | 15% | Clean, well-organized, readable, and properly formatted code | Mostly clean code with minor issues | Code is somewhat unclear or inconsistent | Poorly written, unreadable code |
| Functionality | 20% | Code works perfectly for all test cases | Works for most test cases | Works for limited cases | Does not run or fails major cases |
| Error Handling | 10% | Handles all edge cases and errors effectively | Handles some edge cases | Limited error handling | No error handling |
| Efficiency | 10% | Highly optimized in time and space complexity | Reasonably efficient | Some inefficiencies | Highly inefficient |
| Documentation & Comments | 5% | Clear and meaningful comments and documentation | Adequate comments | Few or unclear comments | No comments |
| Testing & Debugging | 5% | Thoroughly tested with multiple cases; no bugs | Tested with some cases | Minimal testing | No testing done |