SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

Name: Pranav  AS

Reg no: 192573001

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

ANSWERS

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Determine the network address, valid host IP range, broadcast address, and total number of usable host IP addresses for each lab.

Given:

• IP Address Block = 192.168.20.0/24

• Lab A Subnet = /26

• Lab B Subnet = /27

• Lab C Subnet = /28

Step 1: Calculate Lab A (/26)

Subnet Mask = 255.255.255.192

Number of IP Addresses = 2^(32−26)

= 2^6

= 64

Usable Hosts = 64 − 2

= 62 hosts

Block Size = 64

Network Address = 192.168.20.0

Host Range = 192.168.20.1 – 192.168.20.62

Broadcast Address = 192.168.20.63

Answer:

• Network Address = 192.168.20.0

• Host Range = 192.168.20.1 – 192.168.20.62

• Broadcast Address = 192.168.20.63

• Usable Hosts = 62

Step 2: Calculate Lab B (/27)

Next Available Address = 192.168.20.64

Subnet Mask = 255.255.255.224

Number of IP Addresses = 2^(32−27)

= 2^5

= 32

Usable Hosts = 32 − 2

= 30 hosts

Block Size = 32

Network Address = 192.168.20.64

Host Range = 192.168.20.65 – 192.168.20.94

Broadcast Address = 192.168.20.95

Answer:

• Network Address = 192.168.20.64

• Host Range = 192.168.20.65 – 192.168.20.94

• Broadcast Address = 192.168.20.95

• Usable Hosts = 30

Step 3: Calculate Lab C (/28)

Next Available Address = 192.168.20.96

Subnet Mask = 255.255.255.240

Number of IP Addresses = 2^(32−28)

= 2^4

= 16

Usable Hosts = 16 − 2

= 14 hosts

Block Size = 16

Network Address = 192.168.20.96

Host Range = 192.168.20.97 – 192.168.20.110

Broadcast Address = 192.168.20.111

Answer:

• Network Address = 192.168.20.96

• Host Range = 192.168.20.97 – 192.168.20.110

• Broadcast Address = 192.168.20.111

• Usable Hosts = 14

Final Answers:

• Lab A

- Network Address = 192.168.20.0

- Host Range = 192.168.20.1 – 192.168.20.62

- Broadcast Address = 192.168.20.63

- Usable Hosts = 62

• Lab B

- Network Address = 192.168.20.64

- Host Range = 192.168.20.65 – 192.168.20.94

- Broadcast Address = 192.168.20.95

- Usable Hosts = 30

• Lab C

- Network Address = 192.168.20.96

- Host Range = 192.168.20.97 – 192.168.20.110

- Broadcast Address = 192.168.20.111

- Usable Hosts = 14

Explanation

Subnetting divides a large network into multiple smaller logical networks. This improves network performance, reduces broadcast traffic, increases security, and efficiently utilizes available IP addresses.

• Lab A uses a /26 subnet, providing 62 usable host addresses for a larger laboratory.

• Lab B uses a /27 subnet, providing 30 usable host addresses for a medium-sized laboratory.

• Lab C uses a /28 subnet, providing 14 usable host addresses for a smaller laboratory.

Each subnet has its own network address, host range, and broadcast address, ensuring proper communication within each laboratory while preventing unnecessary broadcast traffic.

Conclusion

The 192.168.20.0/24 network is successfully divided into three subnets according to the required subnet masks. Lab A receives 62 usable host addresses, Lab B receives 30 usable host addresses, and Lab C receives 14 usable host addresses. The calculated network addresses, valid host ranges, and broadcast addresses ensure efficient IP address allocation, better network management, and improved communication within the university network.

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

Given:

• System 1 IP Address = 192.168.1.130

• System 2 IP Address = 192.168.1.190

• Subnet Mask = 255.255.255.192 (/26)

Step 1: Calculate the Block Size

Subnet Mask = 255.255.255.192

Block Size = 256 − 192

= 64

The subnet ranges are:

• 192.168.1.0 – 192.168.1.63

• 192.168.1.64 – 192.168.1.127

• 192.168.1.128 – 192.168.1.191

• 192.168.1.192 – 192.168.1.255

Each subnet contains:

Number of IP Addresses = 2^(32−26)

= 2^6

= 64

Usable Hosts = 64 − 2

= 62 hosts

Step 2: Calculate the Details for System 1

IP Address = 192.168.1.130

The IP address lies between:

192.168.1.128 – 192.168.1.191

Therefore,

Network Address = 192.168.1.128

Valid Host Range = 192.168.1.129 – 192.168.1.190

Broadcast Address = 192.168.1.191

Since 192.168.1.130 lies within the valid host range,

Answer:

• Network Address = 192.168.1.128

• Host Range = 192.168.1.129 – 192.168.1.190

• Broadcast Address = 192.168.1.191

• IP Address 192.168.1.130 belongs to this subnet.

• Usable Hosts = 62

Step 3: Calculate the Details for System 2

IP Address = 192.168.1.190

The IP address also lies between:

192.168.1.128 – 192.168.1.191

Therefore,

Network Address = 192.168.1.128

Valid Host Range = 192.168.1.129 – 192.168.1.190

Broadcast Address = 192.168.1.191

Since 192.168.1.190 lies within the valid host range,

Answer:

• Network Address = 192.168.1.128

• Host Range = 192.168.1.129 – 192.168.1.190

• Broadcast Address = 192.168.1.191

• IP Address 192.168.1.190 belongs to this subnet.

• Usable Hosts = 62

Final Answers:

System 1

• Network Address = 192.168.1.128

• Valid Host Range = 192.168.1.129 – 192.168.1.190

• Broadcast Address = 192.168.1.191

• Usable Hosts = 62

• IP Address 192.168.1.130 belongs to the subnet.

System 2

• Network Address = 192.168.1.128

• Valid Host Range = 192.168.1.129 – 192.168.1.190

• Broadcast Address = 192.168.1.191

• Usable Hosts = 62

• IP Address 192.168.1.190 belongs to the same subnet.

Explanation

The subnet mask 255.255.255.192 (/26) divides the original network into four equal subnets, each containing 64 IP addresses and 62 usable host addresses. Both System 1 and System 2 fall within the subnet range 192.168.1.128 – 192.168.1.191. Therefore, both systems belong to the same subnet and can communicate directly without requiring a router. The subnetting process simplifies network management, reduces unnecessary broadcast traffic, and improves overall network performance.

Conclusion

Using the /26 subnet mask, both System 1 (192.168.1.130) and System 2 (192.168.1.190) are identified as members of the subnet 192.168.1.128/26. The network address is 192.168.1.128, the broadcast address is 192.168.1.191, and the valid host range is 192.168.1.129 to 192.168.1.190. Each subnet supports 62 usable host addresses, ensuring efficient IP address utilization and reliable communication between devices.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

Given:

• IP Address Block = 172.16.0.0/16

• HR Department = /25

• Sales Department = /26

• IT Department = /27

• Admin Department = /28

Step 1: Calculate the HR Department (/25)

Subnet Mask = 255.255.255.128

Number of IP Addresses = 2^(32−25)

= 2^7

= 128

Usable Hosts = 128 − 2

= 126 hosts

Network Address = 172.16.0.0

Valid Host Range = 172.16.0.1 – 172.16.0.126

Broadcast Address = 172.16.0.127

Answer:

• Network Address = 172.16.0.0

• Host Range = 172.16.0.1 – 172.16.0.126

• Broadcast Address = 172.16.0.127

• Usable Hosts = 126

Step 2: Calculate the Sales Department (/26)

Next Available Address = 172.16.0.128

Subnet Mask = 255.255.255.192

Number of IP Addresses = 2^(32−26)

= 2^6

= 64

Usable Hosts = 64 − 2

= 62 hosts

Network Address = 172.16.0.128

Valid Host Range = 172.16.0.129 – 172.16.0.190

Broadcast Address = 172.16.0.191

Answer:

• Network Address = 172.16.0.128

• Host Range = 172.16.0.129 – 172.16.0.190

• Broadcast Address = 172.16.0.191

• Usable Hosts = 62

Step 3: Calculate the IT Department (/27)

Next Available Address = 172.16.0.192

Subnet Mask = 255.255.255.224

Number of IP Addresses = 2^(32−27)

= 2^5

= 32

Usable Hosts = 32 − 2

= 30 hosts

Network Address = 172.16.0.192

Valid Host Range = 172.16.0.193 – 172.16.0.222

Broadcast Address = 172.16.0.223

Answer:

• Network Address = 172.16.0.192

• Host Range = 172.16.0.193 – 172.16.0.222

• Broadcast Address = 172.16.0.223

• Usable Hosts = 30

Step 4: Calculate the Admin Department (/28)

Next Available Address = 172.16.0.224

Subnet Mask = 255.255.255.240

Number of IP Addresses = 2^(32−28)

= 2^4

= 16

Usable Hosts = 16 − 2

= 14 hosts

Network Address = 172.16.0.224

Valid Host Range = 172.16.0.225 – 172.16.0.238

Broadcast Address = 172.16.0.239

Answer:

• Network Address = 172.16.0.224

• Host Range = 172.16.0.225 – 172.16.0.238

• Broadcast Address = 172.16.0.239

• Usable Hosts = 14

Step 5: Calculate the Remaining Unused IP Address Range

Allocated Address Range:

• HR      : 172.16.0.0 – 172.16.0.127

• Sales   : 172.16.0.128 – 172.16.0.191

• IT      : 172.16.0.192 – 172.16.0.223

• Admin   : 172.16.0.224 – 172.16.0.239

Remaining Unused Range:

172.16.0.240 – 172.16.255.255

Answer:

• Remaining Unused IP Address Range = 172.16.0.240 – 172.16.255.255

Final Answers:

HR Department

• Network Address = 172.16.0.0

• Valid Host Range = 172.16.0.1 – 172.16.0.126

• Broadcast Address = 172.16.0.127

• Usable Hosts = 126

Sales Department

• Network Address = 172.16.0.128

• Valid Host Range = 172.16.0.129 – 172.16.0.190

• Broadcast Address = 172.16.0.191

• Usable Hosts = 62

IT Department

• Network Address = 172.16.0.192

• Valid Host Range = 172.16.0.193 – 172.16.0.222

• Broadcast Address = 172.16.0.223

• Usable Hosts = 30

Admin Department

• Network Address = 172.16.0.224

• Valid Host Range = 172.16.0.225 – 172.16.0.238

• Broadcast Address = 172.16.0.239

• Usable Hosts = 14

Remaining Unused IP Address Range

• 172.16.0.240 – 172.16.255.255

Explanation

Subnetting divides the large 172.16.0.0/16 network into smaller subnetworks for different departments. Each department receives a subnet based on its host requirements. The HR department receives the largest subnet with 126 usable hosts, followed by Sales with 62 hosts, IT with 30 hosts, and Admin with 14 hosts. The remaining IP addresses are reserved for future expansion, allowing the organization to add more departments or devices without changing the existing network structure.

Conclusion

The IP block 172.16.0.0/16 has been successfully divided into four subnets according to the required subnet masks. Each department has its own network address, valid host range, broadcast address, and sufficient usable host addresses. The remaining IP address range from 172.16.0.240 to 172.16.255.255 remains available for future network expansion, ensuring efficient IP address management and scalable network design.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

Given:

• IP Address Block = 172.16.10.0/24

• Subnet Mask = /27

• IP Address 1 = 172.16.10.78

• IP Address 2 = 172.16.10.95

Step 1: Calculate the Total Number of Subnets

Original Prefix = /24

New Prefix = /27

Borrowed Bits = 27 − 24

= 3 bits

Number of Subnets = 2^3

= 8 subnets

Answer: 8 subnets

Step 2: Calculate the Number of Hosts per Subnet

Host Bits = 32 − 27

= 5

Total IP Addresses = 2^5

= 32

Usable Hosts = 32 − 2

= 30 hosts

Answer: 30 usable hosts per subnet

Step 3: Identify the Subnet for IP Address 172.16.10.78

Subnet Mask = 255.255.255.224

Block Size = 256 − 224

= 32

The subnet ranges are:

• 172.16.10.0   – 172.16.10.31

• 172.16.10.32  – 172.16.10.63

• 172.16.10.64  – 172.16.10.95

• 172.16.10.96  – 172.16.10.127

• 172.16.10.128 – 172.16.10.159

• 172.16.10.160 – 172.16.10.191

• 172.16.10.192 – 172.16.10.223

• 172.16.10.224 – 172.16.10.255

The IP Address 172.16.10.78 falls in the range:

172.16.10.64 – 172.16.10.95

Therefore,

Network Address = 172.16.10.64

Valid Host Range = 172.16.10.65 – 172.16.10.94

Broadcast Address = 172.16.10.95

Answer:

• Network Address = 172.16.10.64

• Host Range = 172.16.10.65 – 172.16.10.94

• Broadcast Address = 172.16.10.95

Step 4: Verify IP Address 172.16.10.95

Given IP Address = 172.16.10.95

Broadcast Address of the subnet = 172.16.10.95

Since 172.16.10.95 is the broadcast address, it cannot be assigned to any host.

Therefore,

172.16.10.95 does not belong to the valid host range.

Answer:

• 172.16.10.95 is the Broadcast Address.

• It is not a valid host IP address.

Final Answers:

• Total Number of Subnets = 8

• Number of Usable Hosts per Subnet = 30

• IP Address 172.16.10.78 belongs to the subnet 172.16.10.64/27

• Network Address = 172.16.10.64

• Valid Host Range = 172.16.10.65 – 172.16.10.94

• Broadcast Address = 172.16.10.95

• IP Address 172.16.10.95 is the Broadcast Address and cannot be assigned to a host.

Explanation

Subnetting with a /27 subnet mask divides the original /24 network into eight equal subnets. Each subnet contains 32 IP addresses, of which 30 are usable for hosts. The IP address 172.16.10.78 belongs to the third subnet (172.16.10.64/27). The broadcast address for this subnet is 172.16.10.95, which is reserved for broadcasting messages to all devices within the subnet and therefore cannot be assigned to any host. Subnetting improves network organization, minimizes broadcast traffic, enhances security, and allows efficient utilization of IP addresses.

Conclusion

The network 172.16.10.0/24 has been successfully divided into eight subnets using the /27 subnet mask. Each subnet supports 30 usable host addresses. The IP address 172.16.10.78 belongs to the subnet 172.16.10.64/27 with the network address 172.16.10.64 and broadcast address 172.16.10.95. Since 172.16.10.95 is the broadcast address, it cannot be used as a host IP address, ensuring proper network communication and efficient subnet management.

5. A network uses the Internet Protocol version 6 (IPv6) address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

Given:

• IPv6 Address = 2001:0db8:0000:0000:0000:ff00:0042:8329

• Prefix Length = /64

Step 1: Compress the IPv6 Address

Original IPv6 Address

= 2001:0db8:0000:0000:0000:ff00:0042:8329

Rules for IPv6 Compression:

• Remove leading zeros from each hexadecimal block.

• Replace one continuous sequence of zero blocks with (::).

Compressed IPv6 Address

= 2001:db8::ff00:42:8329

Answer:

• Compressed IPv6 Address = 2001:db8::ff00:42:8329

Step 2: Expand the Compressed IPv6 Address

Compressed IPv6 Address

= 2001:db8::ff00:42:8329

Expand (::) back into zero blocks and restore leading zeros.

Expanded IPv6 Address

= 2001:0db8:0000:0000:0000:ff00:0042:8329

Answer:

• Expanded IPv6 Address = 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 3: Determine the Network Prefix

Prefix Length

= /64

The first 64 bits (first four hexadecimal blocks) represent the network portion.

Network Prefix

= 2001:0db8:0000:0000::/64

Answer:

• Network Prefix = 2001:0db8:0000:0000::/64

Step 4: Calculate the Total Number of Host Addresses

Host Bits

= 128 − 64

= 64 bits

Total Host Addresses

= 2^64

= 18,446,744,073,709,551,616 host addresses

Answer:

• Total Host Addresses = 18,446,744,073,709,551,616

Final Answers:

• Compressed IPv6 Address = 2001:db8::ff00:42:8329

• Expanded IPv6 Address = 2001:0db8:0000:0000:0000:ff00:0042:8329

• Network Prefix = 2001:0db8:0000:0000::/64

• Total Host Addresses = 18,446,744,073,709,551,616 (2^64)

Explanation

IPv6 uses 128-bit addresses to provide a significantly larger address space than IPv4. Compression rules simplify the representation of IPv6 addresses by removing leading zeros and replacing consecutive zero blocks with (::). The /64 prefix indicates that the first 64 bits represent the network portion, while the remaining 64 bits are reserved for host addresses. This large address space allows billions of devices to be connected while supporting efficient routing, improved scalability, and simplified network management.

Conclusion

The given IPv6 address is successfully compressed to **2001:db8::ff00:42:8329** and expanded back to its original form **2001:0db8:0000:0000:0000:ff00:0042:8329**. Using a **/64** prefix, the network prefix is **2001:0db8:0000:0000::/64**, leaving **64 bits** for host addressing. This provides **2^64 (18,446,744,073,709,551,616)** possible host addresses, demonstrating the vast addressing capability and scalability of IPv6 for modern computer networks.