## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

NAME: PRANAV A S

REG NO: 192573001

COURSE NAME: COMPUTER NETWORKS      COURSE CODE: CSA0716

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26 Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine	the	network	address	and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning ) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned , logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |

ANSWERS

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by determining the network address, broadcast address, valid host IP range, usable host addresses, and remaining unused IP address range.

Given:

• IP Address Block = 192.168.100.0/24

• School of Computing = /25

• School of Electronics = /26

• School of Mechanical Engineering = /27

Step 1: Calculate the School of Computing (/25)

Subnet Mask = 255.255.255.128

Number of IP Addresses = 2^(32−25)

= 2^7

= 128

Usable Hosts = 128 − 2

= 126 hosts

Block Size = 128

Network Address = 192.168.100.0

Valid Host Range = 192.168.100.1 – 192.168.100.126

Broadcast Address = 192.168.100.127

Answer:

• Network Address = 192.168.100.0

• Valid Host Range = 192.168.100.1 – 192.168.100.126

• Broadcast Address = 192.168.100.127

• Usable Hosts = 126

Step 2: Calculate the School of Electronics (/26)

Next Available Address = 192.168.100.128

Subnet Mask = 255.255.255.192

Number of IP Addresses = 2^(32−26)

= 2^6

= 64

Usable Hosts = 64 − 2

= 62 hosts

Block Size = 64

Network Address = 192.168.100.128

Valid Host Range = 192.168.100.129 – 192.168.100.190

Broadcast Address = 192.168.100.191

Answer:

• Network Address = 192.168.100.128

• Valid Host Range = 192.168.100.129 – 192.168.100.190

• Broadcast Address = 192.168.100.191

• Usable Hosts = 62

Step 3: Calculate the School of Mechanical Engineering (/27)

Next Available Address = 192.168.100.192

Subnet Mask = 255.255.255.224

Number of IP Addresses = 2^(32−27)

= 2^5

= 32

Usable Hosts = 32 − 2

= 30 hosts

Block Size = 32

Network Address = 192.168.100.192

Valid Host Range = 192.168.100.193 – 192.168.100.222

Broadcast Address = 192.168.100.223

Answer:

• Network Address = 192.168.100.192

• Valid Host Range = 192.168.100.193 – 192.168.100.222

• Broadcast Address = 192.168.100.223

• Usable Hosts = 30

Step 4: Calculate the Remaining Unused IP Address Range

Allocated Address Range

• School of Computing = 192.168.100.0 – 192.168.100.127

• School of Electronics = 192.168.100.128 – 192.168.100.191

• School of Mechanical Engineering = 192.168.100.192 – 192.168.100.223

Remaining Unused Range

192.168.100.224 – 192.168.100.255

Answer

• Remaining Unused IP Address Range = 192.168.100.224 – 192.168.100.255

Final Answers

School of Computing

• Network Address = 192.168.100.0

• Valid Host Range = 192.168.100.1 – 192.168.100.126

• Broadcast Address = 192.168.100.127

• Usable Hosts = 126

School of Electronics

• Network Address = 192.168.100.128

• Valid Host Range = 192.168.100.129 – 192.168.100.190

• Broadcast Address = 192.168.100.191

• Usable Hosts = 62

School of Mechanical Engineering

• Network Address = 192.168.100.192

• Valid Host Range = 192.168.100.193 – 192.168.100.222

• Broadcast Address = 192.168.100.223

• Usable Hosts = 30

Remaining Unused IP Address Range

• 192.168.100.224 – 192.168.100.255

Explanation

Subnetting divides the 192.168.100.0/24 network into smaller subnetworks for different departments. The School of Computing receives the largest subnet with 126 usable host addresses, followed by the School of Electronics with 62 hosts and the School of Mechanical Engineering with 30 hosts. Each subnet has its own network address, host range, and broadcast address, ensuring efficient communication, reduced broadcast traffic, improved security, and better utilization of IP addresses. The remaining IP addresses are reserved for future expansion.

Conclusion

The Smart Campus network has been successfully subnetted according to the required subnet masks. Each department receives an independent subnet with sufficient host addresses for its devices. The remaining address range from 192.168.100.224 to 192.168.100.255 is available for future network expansion, making the network scalable, organized, and efficient.

ANSWERS

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, determine the network address, broadcast address, valid host IP range, usable host addresses, and remaining unused IP address range for each department.

Given:

• Network Block = 10.10.0.0/16

• Software Development = /24

• Quality Assurance = /25

• Human Resources = /26

• Executive Management = /27

Step 1: Calculate the Software Development Department (/24)

Subnet Mask = 255.255.255.0

Number of IP Addresses = 2^(32−24)

= 2^8

= 256

Usable Hosts = 256 − 2

= 254 hosts

Block Size = 256

Network Address = 10.10.0.0

Valid Host Range = 10.10.0.1 – 10.10.0.254

Broadcast Address = 10.10.0.255

Answer:

• Network Address = 10.10.0.0

• Valid Host Range = 10.10.0.1 – 10.10.0.254

• Broadcast Address = 10.10.0.255

• Usable Hosts = 254

Step 2: Calculate the Quality Assurance Department (/25)

Next Available Address = 10.10.1.0

Subnet Mask = 255.255.255.128

Number of IP Addresses = 2^(32−25)

= 2^7

= 128

Usable Hosts = 128 − 2

= 126 hosts

Block Size = 128

Network Address = 10.10.1.0

Valid Host Range = 10.10.1.1 – 10.10.1.126

Broadcast Address = 10.10.1.127

Answer:

• Network Address = 10.10.1.0

• Valid Host Range = 10.10.1.1 – 10.10.1.126

• Broadcast Address = 10.10.1.127

• Usable Hosts = 126

Step 3: Calculate the Human Resources Department (/26)

Next Available Address = 10.10.1.128

Subnet Mask = 255.255.255.192

Number of IP Addresses = 2^(32−26)

= 2^6

= 64

Usable Hosts = 64 − 2

= 62 hosts

Block Size = 64

Network Address = 10.10.1.128

Valid Host Range = 10.10.1.129 – 10.10.1.190

Broadcast Address = 10.10.1.191

Answer:

• Network Address = 10.10.1.128

• Valid Host Range = 10.10.1.129 – 10.10.1.190

• Broadcast Address = 10.10.1.191

• Usable Hosts = 62

Step 4: Calculate the Executive Management Department (/27)

Next Available Address = 10.10.1.192

Subnet Mask = 255.255.255.224

Number of IP Addresses = 2^(32−27)

= 2^5

= 32

Usable Hosts = 32 − 2

= 30 hosts

Block Size = 32

Network Address = 10.10.1.192

Valid Host Range = 10.10.1.193 – 10.10.1.222

Broadcast Address = 10.10.1.223

Answer:

• Network Address = 10.10.1.192

• Valid Host Range = 10.10.1.193 – 10.10.1.222

• Broadcast Address = 10.10.1.223

• Usable Hosts = 30

Step 5: Calculate the Remaining Unused IP Address Range

Allocated Address Range

• Software Development = 10.10.0.0 – 10.10.0.255

• Quality Assurance = 10.10.1.0 – 10.10.1.127

• Human Resources = 10.10.1.128 – 10.10.1.191

• Executive Management = 10.10.1.192 – 10.10.1.223

Remaining Unused Range

10.10.1.224 – 10.10.255.255

Answer

• Remaining Unused IP Address Range = 10.10.1.224 – 10.10.255.255

Final Answers

Software Development

• Network Address = 10.10.0.0

• Valid Host Range = 10.10.0.1 – 10.10.0.254

• Broadcast Address = 10.10.0.255

• Usable Hosts = 254

Quality Assurance

• Network Address = 10.10.1.0

• Valid Host Range = 10.10.1.1 – 10.10.1.126

• Broadcast Address = 10.10.1.127

• Usable Hosts = 126

Human Resources

• Network Address = 10.10.1.128

• Valid Host Range = 10.10.1.129 – 10.10.1.190

• Broadcast Address = 10.10.1.191

• Usable Hosts = 62

Executive Management

• Network Address = 10.10.1.192

• Valid Host Range = 10.10.1.193 – 10.10.1.222

• Broadcast Address = 10.10.1.223

• Usable Hosts = 30

Remaining Unused IP Address Range

• 10.10.1.224 – 10.10.255.255

Explanation

The allocated network block 10.10.0.0/16 has been divided into multiple subnets based on the host requirements of each department. Software Development receives the largest subnet with 254 usable host addresses, followed by Quality Assurance with 126 hosts, Human Resources with 62 hosts, and Executive Management with 30 hosts. Each subnet has a unique network address, host range, and broadcast address, ensuring efficient communication, better network performance, reduced broadcast traffic, and easier network administration. The remaining IP address range is reserved for future departmental expansion.

Conclusion

The network has been successfully subnetted according to the required subnet masks. Each department has sufficient IP addresses for its devices while maintaining efficient IP address utilization. The remaining address range from 10.10.1.224 to 10.10.255.255 can be used for future expansion, ensuring scalability and effective network management.

3. A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following tasks:

1. Convert the given IPv6 address into its compressed notation.

2. Expand the compressed address back into its full hexadecimal representation.

3. Identify the network prefix using the /64 prefix length.

4. Calculate the total number of host addresses supported within the subnet.

Given:

• IPv6 Address = 2001:0db8:abcd:0000:0000:0000:0000:0000/64

• Prefix Length = /64

Step 1: Convert the IPv6 Address into Compressed Notation

Rules for IPv6 Compression

• Remove all leading zeros in each hexadecimal block.

• Replace one longest sequence of consecutive "0000" blocks with "::".

• "::" can be used only once in an IPv6 address.

Original Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Compressed Address

2001:db8:abcd::/64

Answer

• Compressed IPv6 Address = 2001:db8:abcd::/64

Step 2: Expand the Compressed Address

Compressed Address

2001:db8:abcd::/64

Expanding the "::" back to all omitted hexadecimal blocks gives:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Answer

• Expanded IPv6 Address = 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Step 3: Identify the Network Prefix

Prefix Length = /64

The first 64 bits represent the Network ID.

The remaining 64 bits represent the Host ID.

Network Prefix

2001:db8:abcd:0::/64

Answer

• Network Prefix = 2001:db8:abcd:0::/64

• Network Portion = First 64 bits

• Host Portion = Last 64 bits

Step 4: Calculate the Total Number of Host Addresses

IPv6 Address Length = 128 bits

Network Prefix = 64 bits

Host Bits = 128 − 64

= 64 bits

Total Host Addresses

= 2^64

= 18,446,744,073,709,551,616

Answer

• Total Host Addresses = 18,446,744,073,709,551,616

Final Answers

Compressed IPv6 Address

• 2001:db8:abcd::/64

Expanded IPv6 Address

• 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Network Prefix

• 2001:db8:abcd:0::/64

Host Portion

• Last 64 bits

Total Host Addresses

• 18,446,744,073,709,551,616

Explanation

IPv6 uses 128-bit addresses, providing an extremely large address space compared to IPv4. Compression removes leading zeros and replaces consecutive groups of zero blocks with "::", making the address easier to read and manage. With a /64 prefix, the first 64 bits identify the network while the remaining 64 bits are reserved for host addresses. This allows the network to support over 18 quintillion unique devices, making IPv6 ideal for smart hospitals, IoT sensors, medical equipment, servers, and future network expansion.

Conclusion

The given IPv6 address has been successfully compressed and expanded while preserving its original value. The /64 network prefix correctly separates the network and host portions. The subnet supports 18,446,744,073,709,551,616 host addresses, ensuring excellent scalability and sufficient address space for current and future smart hospital devices.

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

1. The destination subnet to which the packet belongs.

2. The matching routing table entry.

3. The next-hop network/interface used to forward the packet.

4. The default route that should be used if the destination does not match any routing table entry.

Given:

• Routing Table Entries:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

• Destination IP Address = 192.168.2.45

Step 1: Determine the Destination Subnet

Destination IP Address

192.168.2.45

Subnet Mask

255.255.255.0 (/24)

The first three octets of the destination IP are:

192.168.2

Therefore, the packet belongs to the subnet:

192.168.2.0/24

Answer

• Destination Subnet = 192.168.2.0/24

Step 2: Identify the Matching Routing Table Entry

Routing Table

• 192.168.1.0/24

• 192.168.2.0/24

• 192.168.3.0/24

The destination IP matches the routing entry:

192.168.2.0/24

Answer

• Matching Routing Table Entry = 192.168.2.0/24

Step 3: Determine the Next-Hop Network/Interface

Since the destination belongs to the network 192.168.2.0/24, the router forwards the packet through the interface connected to this subnet.

If a next-hop router is configured, the packet will be forwarded to that router for further delivery.

Answer

• Next-Hop Interface = Interface connected to 192.168.2.0/24 network

• Next-Hop Router = Configured router for the 192.168.2.0/24 subnet (if applicable)

Step 4: Determine the Default Route

If the destination IP address does not match any routing table entry, the router forwards the packet using the default route.

Default Route

0.0.0.0/0

The default route directs unknown destinations to the default gateway.

Answer

• Default Route = 0.0.0.0/0

Final Answers

Destination IP Address

• 192.168.2.45

Destination Subnet

• 192.168.2.0/24

Matching Routing Table Entry

• 192.168.2.0/24

Next-Hop Interface

• Interface connected to the 192.168.2.0/24 network

Default Route

• 0.0.0.0/0

Explanation

A router forwards packets by comparing the destination IP address with the entries in its routing table. Since the destination address 192.168.2.45 belongs to the 192.168.2.0/24 network, the router selects the corresponding routing table entry and forwards the packet through the interface connected to that subnet. If no matching route is found, the router uses the default route (0.0.0.0/0) to send the packet to the default gateway, ensuring that communication can continue even when a specific route is unavailable.

Conclusion

The destination IP address 192.168.2.45 successfully matches the routing table entry 192.168.2.0/24. Therefore, the packet is forwarded through the interface connected to that network. If no matching network existed, the router would forward the packet using the default route 0.0.0.0/0, enabling reliable packet delivery across different networks.

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each LAN.

2. Identify the valid host IP address range.

3. Suggest suitable IP addresses for three computers in each LAN.

4. Identify the default gateway that hosts in each LAN should use for communication.

Given:

• Administration LAN = 192.168.10.0/26

• Finance LAN = 192.168.10.64/26

• Administration Gateway = 192.168.10.1

• Finance Gateway = 192.168.10.65

Step 1: Determine the Network Address and Broadcast Address

Administration LAN

Subnet Mask = 255.255.255.192

Block Size = 64 IP Addresses

Network Address = 192.168.10.0

Broadcast Address = 192.168.10.63

Finance LAN

Subnet Mask = 255.255.255.192

Block Size = 64 IP Addresses

Network Address = 192.168.10.64

Broadcast Address = 192.168.10.127

Answer

Administration LAN

• Network Address = 192.168.10.0

• Broadcast Address = 192.168.10.63

Finance LAN

• Network Address = 192.168.10.64

• Broadcast Address = 192.168.10.127

Step 2: Determine the Valid Host IP Address Range

Administration LAN

First Host = 192.168.10.1

Last Host = 192.168.10.62

Host Range = 192.168.10.1 – 192.168.10.62

Finance LAN

First Host = 192.168.10.65

Last Host = 192.168.10.126

Host Range = 192.168.10.65 – 192.168.10.126

Answer

Administration LAN

• Host Range = 192.168.10.1 – 192.168.10.62

Finance LAN

• Host Range = 192.168.10.65 – 192.168.10.126

Step 3: Suggest Suitable IP Addresses for Three Computers

Administration LAN

PC1 = 192.168.10.2

PC2 = 192.168.10.3

PC3 = 192.168.10.4

Finance LAN

PC1 = 192.168.10.66

PC2 = 192.168.10.67

PC3 = 192.168.10.68

Answer

Administration LAN

• PC1 = 192.168.10.2

• PC2 = 192.168.10.3

• PC3 = 192.168.10.4

Finance LAN

• PC1 = 192.168.10.66

• PC2 = 192.168.10.67

• PC3 = 192.168.10.68

Step 4: Identify the Default Gateway

Administration LAN

Default Gateway = 192.168.10.1

Finance LAN

Default Gateway = 192.168.10.65

Answer

Administration LAN

• Default Gateway = 192.168.10.1

Finance LAN

• Default Gateway = 192.168.10.65

Final Answers

Administration LAN

• Network Address = 192.168.10.0

• Broadcast Address = 192.168.10.63

• Valid Host Range = 192.168.10.1 – 192.168.10.62

• Default Gateway = 192.168.10.1

• Suggested Computer IP Addresses:

- PC1 = 192.168.10.2

- PC2 = 192.168.10.3

- PC3 = 192.168.10.4

Finance LAN

• Network Address = 192.168.10.64

• Broadcast Address = 192.168.10.127

• Valid Host Range = 192.168.10.65 – 192.168.10.126

• Default Gateway = 192.168.10.65

• Suggested Computer IP Addresses:

- PC1 = 192.168.10.66

- PC2 = 192.168.10.67

- PC3 = 192.168.10.68

Explanation

The company network is divided into two separate /26 subnets, one for the Administration department and another for the Finance department. Each subnet contains 64 IP addresses, with 62 usable host addresses after excluding the network and broadcast addresses. The router provides a separate default gateway for each LAN, allowing hosts to communicate both within their own subnet and with other networks. Suitable IP addresses are assigned to the computers from the valid host range while avoiding the network, broadcast, and gateway addresses.

Conclusion

The company network has been successfully configured into two independent LANs with their own network addresses, broadcast addresses, valid host ranges, default gateways, and computer IP addresses. This subnet design improves network organization, security, and communication efficiency while allowing both departments to communicate through the Layer-3 router.